# About Sophos XG Firewall

Sophos XG Firewall is a next-generation firewall that is packaged as an all-in-one solution. It combines advanced networking capabilities, as well as protections such as Intrusion Prevention Systems (IPS) and Web Application Firewall (WAF), plus user and application controls. It is designed to help you protect your Azure-based workloads against advanced threats.

# Objective

Sophos XG Firewall on Azure is a solution to run XG Firewall on Microsoft Azure.

The specific use case for this scenario highlights the Sophos XG WAF component. A customer will launch the Test Drive. Azure Resource Manager Templates will deploy an XG firewall, as well as a Windows Virtual Machine (VM). The customer can log in remotely to the XG Firewall as view the preconfigured security policy. The policy will also provide web filtering and blocked sites, so customers can test acceptable usage policy enforcement (e.g. no porn, gambling, known malicious web sites) of Remote Desktop users.

# Architecture of Sophos XG Firewall

The architecture describes how you can deploy and run Sophos XG Firewall on Azure.

On deploying ARM template, a Vnet is created with 2 subnets (WAN and LAN).

XG Firewall VM is associated with WAN subnet and windows VM with LAN subnet. Firewall rules are added to the Sophos XG Firewall NIC,that allow both inbound and outbound traffic and to allow backend access. The outbound traffic from windows VM goes to XG Firewall and is send through the LAN subnet using route table. Inbound traffic should go directly to Windows VM via XG Firewall.

# Configuring Network Wizard

- Copy the IP address of the Sophos XG Firewall and enter in new browser. Click on ADVANCED.

- This navigates to Sophos admin console. Enter your Sophos credentials with Username as admin.

- Click on Start, this opens the network configuration wizard to perform basic system setup.

- We get to select XG Firewall to run in Gateway or Bridge mode.

- The azure currently doesn’t support bridge mode, So select Gateway Mode.

- Make sure that both interfaces of XG Firewall are configured to Obtain an IP from DHCP. The Gateway name of interface can be changed.

- Configure DNS settings on the XG Firewall to Obtain DNS from DHCP which is a default setting and give the Hostname and enter the Description for easy reference.

- Select policy for LAN to WAN traffic.

- Select default Web Filter template from the list as Allow All.

- Select App Filter, which is necessary for your application such as Blocking generally unwanted application by allowing the rest.

- For IPS, which determines the level and strictness of IPS filtering applied to traffic. Select None.

- The Mail Server Configuration page appears where XG Firewall sends the important notification of significant events.

- In Send Notification to Email Address, give the recipient email address.

- Give windows server IP address in Mail Server IPv4 Address/FQDN and set to default port.

- Followed by email address from where the XG Firewall will send emails in From Email Address.

- Set the Connection security to None.

- Select the Time Zone from drop down menu and manually select date and time.

- Check the Automatically synchronous with NTP Server.

- Sophos recommend the using NTP for time synchronization when using timing critical protocol such as Microsoft server Active Directory. Select Use pre-configured NTP server.

- The very last setup of wizard gives overview of settings configured through the wizard before committing them to system. A quick summary is shown if any changes are needed. Click OK to proceed and commit.

- Progress indicator is shown ,this process takes several minutes. A success page appears which shows that the settings are deployed to the XG Firewall.

- Reload the admin console with the login credentials used earlier which leads to XG Dash board. Your XG Firewall is ready to use.

# Configuration of XG Firewall for Specific Use Case

### The following section describes the configuration steps for RDP access and Blocking Facebook website.

## 4.1. Configuration for Creating RDP access to Windows server

- Log in to your XG Firewall.

- Navigate to Protect > Firewall, click Add Firewall Rule.

- Select Business Application Rule for RDP access. The Add Business Application Rule page opens.

- Make the following settings:

- Application Template: Select DNAT/Full NAT/Load Balancing.

- Mention the name of the Business Rule.

- Source: Select Source Zones as LAN and Allowed Client Networks as Any.

- Destination & Service: Select Destination Host Network as the WAN port i.e Port B and Services Port(s) Forwarded to 3389.

- Forwarded To: In Protected servers, click on Create New and click on IP Address, give windows server IP address. Select the Protected Zone as LAN.

- Give a Name for the IP Host and mention the Private IP address of Window server in IP Address.

- Check the Log Firewall Traffic.

- Check for the created Firewall Rule.

## Configuration To BLock the WEBSITE

- Before configuring Web policy, we can RDP into Firewall VM using windows server credentials and check for Facebook access.

- Log in to your XG Firewall.

- Create a Custom Web Category.

- Navigate to Protect>Web. Go to Categories and click Add to create a new category.

- Give the Name for Web category with Unproductive as Classification. Mention the website which is required to be blocked in Domain.

- A web Category is created on name of Facebook.

- Create a Web Filter Policy to Block Facebook.

- Go to Policies and Click on Add Policy.

- Create a new Web Filter Policy named Block_Facebook.

- Click on Add Rule to add a Web Filter Policy Rule with the following parameters.

Action: Select Block from drop down list and choose Block HTTPS.

Constraints: Select All the Time.

- In Advanced Settings make sure that the Enable logging and reporting is checked and click on Save.

- A web Policy ‘Block_Facebook’ has been created successfully.

- Apply the Policy to a Firewall Rule or User/User Group.

- To create a Rule navigate to Firewall, click on Add Firewall Rule and select User/network Rule.

- Make the following settings

Rule Name: Give the rule name as Block_facebook_rule.

Source: Select Source Zones as LAN from drop down list and Source Networks and       Devices as Any.

Destination & Services: Select Destination Zones as WAN and Destination Networks as Any.

- In Advanced, Select the Web Policy name given earlier and select the Primary Gateway, which was given during network configuration wizard. Check the Log Firewall Traffic and click on Save.

- A Firewall rule ‘Block_facebook_rule’ is created.

# Access RDP and check for Blocked website

- RDP into Firewall VM using windows server credentials.

- Browse the website www.facebook.com in a browser and check .The page will not be displayed as the web policy and a firewall is created.

- We can check the log in by clicking on Log Viewer.

- The above documentation shows that the Sophos XG Firewall can block access to a particular website.

- Similarly the procedure can be followed to block any website.